UE1 Lab report

IN-BCN

BIS BB24

Daniel KARAS, is241323

Stand: 01. Juni 2025

Table of Contents

1	Exercise network plan	3

2	UE1.6) Analysis of RIP	4

Vulnerabilities of RIPng:	8

Hardening mechanisms	8

3	UE1.8) Analysis OSPFv2	9

4	UE1.10) Analysis OSPFv3	12

5	UE1.12) Analysis Multiarea OSPF Dual Stack and Route Redistribution	14

6	UE1.14) Analysis Multiarea OSPF Dual Stack – Stub & Totally Stub Area	17

7	References	19

# Exercise network plan

# UE1.6) Analysis of RIP

- Analyze and record the routing tables of routers RHQ0 and REast1

(Note: Codes are shown only the first time for better readability)

RHQ0# sh ip route

Codes: L - local, C - connected, S - static, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, m - OMP

n - NAT, Ni - NAT inside, No - NAT outside, Nd - NAT DIA

i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2

ia - IS-IS inter area, * - candidate default, U - per-user static route

H - NHRP, G - NHRP registered, g - NHRP registration summary

o - ODR, P - periodic downloaded static route, l - LISP

a - application route

+ - replicated route, % - next hop override, p - overrides from PfR

& - replicated local route overrides by connected

S*    0.0.0.0/0 [1/0] via 199.1.1.65

R     172.16.0.0/16 [120/2] via 172.31.255.254, 00:00:17, Ethernet0/1

R     172.17.0.0/16 [120/2] via 172.31.255.254, 00:00:17, Ethernet0/1

R     172.18.0.0/16 [120/2] via 172.31.255.250, 00:00:04, Ethernet0/2

R     172.19.0.0/16 [120/2] via 172.31.255.250, 00:00:04, Ethernet0/2

172.30.0.0/16 is variably subnetted, 2 subnets, 2 masks

C        172.30.0.0/16 is directly connected, Ethernet0/0

L        172.30.255.254/32 is directly connected, Ethernet0/0

172.31.0.0/16 is variably subnetted, 9 subnets, 2 masks

R        172.31.255.228/30 [120/2] via 172.31.255.254, 00:00:17, Ethernet0/1

R        172.31.255.232/30 [120/1] via 172.31.255.250, 00:00:04, Ethernet0/2

R        172.31.255.236/30 [120/1] via 172.31.255.250, 00:00:04, Ethernet0/2

R        172.31.255.240/30 [120/1] via 172.31.255.254, 00:00:17, Ethernet0/1

R        172.31.255.244/30 [120/1] via 172.31.255.254, 00:00:17, Ethernet0/1

C        172.31.255.248/30 is directly connected, Ethernet0/2

L        172.31.255.249/32 is directly connected, Ethernet0/2

C        172.31.255.252/30 is directly connected, Ethernet0/1

L        172.31.255.253/32 is directly connected, Ethernet0/1

199.1.1.0/24 is variably subnetted, 2 subnets, 2 masks

C        199.1.1.64/30 is directly connected, Ethernet1/0

L        199.1.1.66/32 is directly connected, Ethernet1/0

RHQ0# sh ipv6 route

S   ::/0 [1/0]

via 2001:DB8:ACAD:1::1

C   2001:DB8:ACAD:1::/127 [0/0]

via Ethernet1/0, directly connected

L   2001:DB8:ACAD:1::/128 [0/0]

via Ethernet1/0, receive

R   2001:DB8:ACAD:AC16::/64 [120/3]

via FE80::1, Ethernet0/1

R   2001:DB8:ACAD:AC17::/64 [120/3]

via FE80::1, Ethernet0/1

R   2001:DB8:ACAD:AC18::/64 [120/3]

via FE80::1, Ethernet0/2

R   2001:DB8:ACAD:AC19::/64 [120/3]

via FE80::1, Ethernet0/2

C   2001:DB8:ACAD:AC1E::/64 [0/0]

via Ethernet0/0, directly connected

L   2001:DB8:ACAD:AC1E::254/128 [0/0]

via Ethernet0/0, receive

L   FF00::/8 [0/0]

via Null0, receive

REast1# sho ip route

R*    0.0.0.0/0 [120/1] via 172.31.255.253, 00:00:15, Ethernet0/0

R     172.16.0.0/16 [120/1] via 172.31.255.246, 00:00:16, Ethernet0/1

R     172.17.0.0/16 [120/1] via 172.31.255.242, 00:00:04, Ethernet0/2

R     172.18.0.0/16 [120/3] via 172.31.255.253, 00:00:15, Ethernet0/0

R     172.19.0.0/16 [120/3] via 172.31.255.253, 00:00:15, Ethernet0/0

R     172.30.0.0/16 [120/1] via 172.31.255.253, 00:00:15, Ethernet0/0

172.31.0.0/16 is variably subnetted, 10 subnets, 2 masks

R        172.31.255.228/30 [120/1] via 172.31.255.246, 00:00:16, Ethernet0/1

[120/1] via 172.31.255.242, 00:00:04, Ethernet0/2

R        172.31.255.232/30 [120/2] via 172.31.255.253, 00:00:15, Ethernet0/0

R        172.31.255.236/30 [120/2] via 172.31.255.253, 00:00:15, Ethernet0/0

C        172.31.255.240/30 is directly connected, Ethernet0/2

L        172.31.255.241/32 is directly connected, Ethernet0/2

C        172.31.255.244/30 is directly connected, Ethernet0/1

L        172.31.255.245/32 is directly connected, Ethernet0/1

R        172.31.255.248/30 [120/1] via 172.31.255.253, 00:00:15, Ethernet0/0

C        172.31.255.252/30 is directly connected, Ethernet0/0

L        172.31.255.254/32 is directly connected, Ethernet0/0

REast1# sh ipv6 route

R   ::/0 [120/2]

via FE80::, Ethernet0/0

R   2001:DB8:ACAD:AC16::/64 [120/2]

via FE80::1:2, Ethernet0/1

R   2001:DB8:ACAD:AC17::/64 [120/2]

via FE80::1:3, Ethernet0/2

R   2001:DB8:ACAD:AC18::/64 [120/4]

via FE80::, Ethernet0/0

R   2001:DB8:ACAD:AC19::/64 [120/4]

via FE80::, Ethernet0/0

R   2001:DB8:ACAD:AC1E::/64 [120/2]

via FE80::, Ethernet0/0

L   FF00::/8 [0/0]

via Null0, receive

From reviewing the tables, you can expect the RHQ0 router to be a hub router;
it has Internet access and multiple point to point links.
REast1 is likely to be a branch router, because it relies on the default router of RHQ0.

Static routes (RHQ0) and RIP/ng are in use and can be seen in the tables, for the RIP/ng-learned routes, you can see the administrative distance and metric [ad=120/metric=1-4], in comparison, you can see [0/0] in ipv6 routing tables for the directly connected networks/interfaces: those will always be used first, because these are the lowest(=best) values for ad/metric.

  - How many routes were learned?

RHQ0 has lerned 9 IPv4 RIP routes and 4 IPv6 routes,

REast1 has learned 10 IPv4 RIP routes (including the default route) and 6 IPv6 routes.

- What is the administrative distance of RIP? Where can you find this value on the router?

Using the command

REast1# show ip router rip

…you can see the administrative distance for rip in the brackets next to the network,
the second value is the metric within the same routing protocol.

Figure 1: Administrative distance of IPv4 default route, learned via RIP

- How could you reduce the size of the routing tables?

Multiple options are available, for example:

- Using default routes

- Route aggregation / supernetting [1]

- Filtering route redistribution [2]

- Area based routing [3]

- Which timers does RIP utilize?

“The routing information protocol uses the following timers as part of its operation: …

- Update Timer

  - The update timer controls the interval between two gratuitous Response Message. By default the value is 30 seconds. The response message is broadcast to all its RIP enabled interface.

- Invalid Timer

  - The invalid timer specifies how long a routing entry can be in the routing table without being updated. This is also called as expiration Timer. By default, the value is 180 seconds. After the timer expires the hop count of the routing entry will be set to 16, marking the destination as unreachable.

- Flush Timer

  - The flush timer controls the time between the route is invalidated or marked as unreachable and removal of entry from the routing table. By default the value is 240 seconds. This is 60 seconds longer than Invalid timer. So for 60 seconds the router will be advertising about this unreachable route to all its neighbours. This timer must be set to a higher value than the invalid timer.

- Hold-down Timer

The hold-down timer is started per route entry, when the hop count is changing from lower value to higher value. This allows the route to get stabilized. During this time no update can be done to that routing entry. This is not part of the RFC 1058. This is Cisco's implementation. The default value of this timer is 180 seconds.“

(Timers, [4])

- How can the current values of these timers be displayed?

Using the command

show ip protocols

…you can see the configured timers and, in a table, the last update for learned networks.

Figure 2: RIP timers

- Do some research on how to harden RIPng

### Vulnerabilities of RIPng:

1. RIPng lacks built-in authentication mechanisms,

Packets can be spoofed or injected without verification
(RFC 2081, 3.4 [5])

2. Clear-Text Protocol: All routing information transmitted in plain text, relying on IPSec
(RFC 2080, 4 [6])

3. FF02::9 multicast is used for all updates (UDP 521)

Information leakage to unintended recipients
(RFC 2080, 2.1 [6])

### Hardening mechanisms

1. IPSec Authentication

Configure IPv6 AH/ESP for packet authentication
(referred example vendor ‘HUAWEI’, product NE5000E router; [7])

2. Encryption using IPv6 integrated mechanisms (Next-Header-Field)
(referred example vendor ‘HUAWEI’, product NE40E-F universal service router; [8])

3. Configuring RIPng route control / using ACL)

Filter RIPng traffic on UDP port 521 [9]

3. Distribute Lists

Control route advertisements with IPv6 prefix-lists [10]

  - Ensure that RIPng updates are sent out only on point-to-point links only

This is configured using the following command on non-point-to-point interfaces:

router rip

passive-interface e0/0

# UE1.8) Analysis OSPFv2

- Analyze the routing table of the routers RHQ0 and REast1. Which protocol is used to populate the entries in the routing table now?

RHQ0#sh ip route

S*    0.0.0.0/0 [1/0] via 199.1.1.65

R     172.16.0.0/16 [120/2] via 172.31.255.254, 00:00:09, Ethernet0/1

R     172.17.0.0/16 [120/2] via 172.31.255.254, 00:00:09, Ethernet0/1

R     172.18.0.0/16 [120/2] via 172.31.255.250, 00:00:00, Ethernet0/2

R     172.19.0.0/16 [120/2] via 172.31.255.250, 00:00:00, Ethernet0/2

172.30.0.0/16 is variably subnetted, 2 subnets, 2 masks

C        172.30.0.0/16 is directly connected, Ethernet0/0

L        172.30.255.254/32 is directly connected, Ethernet0/0

172.31.0.0/16 is variably subnetted, 9 subnets, 2 masks

R        172.31.255.228/30 [120/2] via 172.31.255.254, 00:00:09, Ethernet0/1

R        172.31.255.232/30 [120/1] via 172.31.255.250, 00:00:00, Ethernet0/2

R        172.31.255.236/30 [120/1] via 172.31.255.250, 00:00:00, Ethernet0/2

R        172.31.255.240/30 [120/1] via 172.31.255.254, 00:00:09, Ethernet0/1

R        172.31.255.244/30 [120/1] via 172.31.255.254, 00:00:09, Ethernet0/1

C        172.31.255.248/30 is directly connected, Ethernet0/2

L        172.31.255.249/32 is directly connected, Ethernet0/2

C        172.31.255.252/30 is directly connected, Ethernet0/1

L        172.31.255.253/32 is directly connected, Ethernet0/1

199.1.1.0/24 is variably subnetted, 2 subnets, 2 masks

C        199.1.1.64/30 is directly connected, Ethernet1/0

L        199.1.1.66/32 is directly connected, Ethernet1/0

REast1#sh ip route

O*E2  0.0.0.0/0 [110/1] via 172.31.255.253, 00:37:13, Ethernet0/0

R     172.16.0.0/16 [120/1] via 172.31.255.246, 00:00:05, Ethernet0/1

R     172.17.0.0/16 [120/1] via 172.31.255.242, 00:00:22, Ethernet0/2

R     172.18.0.0/16 [120/3] via 172.31.255.253, 00:00:24, Ethernet0/0

R     172.19.0.0/16 [120/3] via 172.31.255.253, 00:00:24, Ethernet0/0

R     172.30.0.0/16 [120/1] via 172.31.255.253, 00:00:24, Ethernet0/0

172.31.0.0/16 is variably subnetted, 10 subnets, 2 masks

R        172.31.255.228/30 [120/1] via 172.31.255.246, 00:00:05, Ethernet0/1

[120/1] via 172.31.255.242, 00:00:22, Ethernet0/2

R        172.31.255.232/30 [120/2] via 172.31.255.253, 00:00:24, Ethernet0/0

R        172.31.255.236/30 [120/2] via 172.31.255.253, 00:00:24, Ethernet0/0

C        172.31.255.240/30 is directly connected, Ethernet0/2

L        172.31.255.241/32 is directly connected, Ethernet0/2

C        172.31.255.244/30 is directly connected, Ethernet0/1

L        172.31.255.245/32 is directly connected, Ethernet0/1

O        172.31.255.248/30 [110/20] via 172.31.255.253, 00:37:14, Ethernet0/0

C        172.31.255.252/30 is directly connected, Ethernet0/0

L        172.31.255.254/32 is directly connected, Ethernet0/0

For router REast1, default route and the point-to-point network between RHQ0 and RWest1 are new learned via OSPF.

In terms of the additional codes to the OSPF-“O”, there is helpful official cisco routing table documentation: [11]

- Analyze the OSPF topology table and the neighbor table of RHQ0 and REast1. (Hint: you will find the commands for that in the OSPF lecture slides)

RHQ0#sh ip ospf da

OSPF Router with ID (1.1.0.0) (Process ID 10)

Router Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum Link count

1.1.0.0         1.1.0.0         8           0x80000004 0x00F4F1 2

1.1.1.1         1.1.1.1         18          0x8000000D 0x00AAD3 1

1.1.2.1         1.1.2.1         18          0x8000000A 0x003851 1

Net Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum

172.31.255.250  1.1.2.1         17          0x80000005 0x009ACB

172.31.255.254  1.1.1.1         17          0x80000005 0x0070F3

Type-5 AS External Link States

Link ID         ADV Router      Age         Seq#       Checksum Tag

0.0.0.0         1.1.0.0         16          0x80000001 0x00CCDA 10

172.31.255.240  1.1.1.1         925         0x80000002 0x0012CF 0

172.31.255.244  1.1.1.1         925         0x80000002 0x00E9F3 0

RHQ0#sho ip ospf nei

Neighbor ID     Pri   State           Dead Time   Address         Interface

1.1.1.1           1   FULL/DR         00:00:37    172.31.255.254  Ethernet0/1

1.1.2.1           1   FULL/DR         00:00:33    172.31.255.250  Ethernet0/2

REast1#sho ip osp da

OSPF Router with ID (1.1.1.1) (Process ID 10)

Router Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum Link count

1.1.0.0         1.1.0.0         74          0x80000004 0x00F4F1 2

1.1.1.1         1.1.1.1         82          0x8000000D 0x00AAD3 1

1.1.2.1         1.1.2.1         83          0x8000000A 0x003851 1

Net Link States (Area 0)

Link ID         ADV Router      Age         Seq#       Checksum

172.31.255.250  1.1.2.1         83          0x80000005 0x009ACB

172.31.255.254  1.1.1.1         82          0x80000005 0x0070F3

Type-5 AS External Link States

Link ID         ADV Router      Age         Seq#       Checksum Tag

0.0.0.0         1.1.0.0         82          0x80000001 0x00CCDA 10

172.31.255.240  1.1.1.1         988         0x80000002 0x0012CF 0

172.31.255.244  1.1.1.1         988         0x80000002 0x00E9F3 0

REast1#sho ip os nei

Neighbor ID     Pri   State           Dead Time   Address         Interface

1.1.0.0           1   FULL/BDR        00:00:34    172.31.255.253  Ethernet0/0

Both routers have the same Router Link States(Router IDs: 1.1.0.0 for RHQ0, 1.1.1.1 for REast1 and 1.1.2.1 for RWest1). ADV Router stands for advertising router (the one, that originated the LSA), Seq# indicates the most recent version of the LSA.

RHQ0 has 2 links (Link count) and REast1/RWest1 both have 1, which indicates, that they are connected to RHQ0. This is also confirmed with the show neighbor commands.

In the neighbor table of RHQ0, you can see that both branch routers were elected Designated Routers (and RHQ0 Backup DR), this is because of the higher router ID; priority (Pri) is 1 (default) on all routers, so the router ID is, what counts in the election process (Page 17 [12]).

# UE1.10) Analysis OSPFv3

a) Analyze the routing table of the routers RHQ0 and REast1. Which protocol is used to populate the entries in the routing table now?

(Note: added IPv4 routing table for completeness)

RHQ0#sh ip rou

S*    0.0.0.0/0 [1/0] via 199.1.1.65

R     172.16.0.0/16 [120/2] via 172.31.255.254, 00:00:11, Ethernet0/1

R     172.17.0.0/16 [120/2] via 172.31.255.254, 00:00:11, Ethernet0/1

R     172.18.0.0/16 [120/2] via 172.31.255.250, 00:00:11, Ethernet0/2

R     172.19.0.0/16 [120/2] via 172.31.255.250, 00:00:11, Ethernet0/2

172.30.0.0/16 is variably subnetted, 2 subnets, 2 masks

C        172.30.0.0/16 is directly connected, Ethernet0/0

L        172.30.255.254/32 is directly connected, Ethernet0/0

172.31.0.0/16 is variably subnetted, 9 subnets, 2 masks

R        172.31.255.228/30 [120/2] via 172.31.255.254, 00:00:11, Ethernet0/1

R        172.31.255.232/30 [120/1] via 172.31.255.250, 00:00:11, Ethernet0/2

R        172.31.255.236/30 [120/1] via 172.31.255.250, 00:00:11, Ethernet0/2

R        172.31.255.240/30 [120/1] via 172.31.255.254, 00:00:11, Ethernet0/1

R        172.31.255.244/30 [120/1] via 172.31.255.254, 00:00:11, Ethernet0/1

C        172.31.255.248/30 is directly connected, Ethernet0/2

L        172.31.255.249/32 is directly connected, Ethernet0/2

C        172.31.255.252/30 is directly connected, Ethernet0/1

L        172.31.255.253/32 is directly connected, Ethernet0/1

199.1.1.0/24 is variably subnetted, 2 subnets, 2 masks

C        199.1.1.64/30 is directly connected, Ethernet1/0

L        199.1.1.66/32 is directly connected, Ethernet1/0

RHQ0#sh ipv rou

S   ::/0 [1/0]

via 2001:DB8:ACAD:1::1

C   2001:DB8:ACAD:1::/127 [0/0]

via Ethernet1/0, directly connected

L   2001:DB8:ACAD:1::/128 [0/0]

via Ethernet1/0, receive

R   2001:DB8:ACAD:AC16::/64 [120/3]

via FE80::1, Ethernet0/1

R   2001:DB8:ACAD:AC17::/64 [120/3]

via FE80::1, Ethernet0/1

R   2001:DB8:ACAD:AC18::/64 [120/3]

via FE80::1, Ethernet0/2

R   2001:DB8:ACAD:AC19::/64 [120/3]

via FE80::1, Ethernet0/2

C   2001:DB8:ACAD:AC1E::/64 [0/0]

via Ethernet0/0, directly connected

L   2001:DB8:ACAD:AC1E::254/128 [0/0]

via Ethernet0/0, receive

L   FF00::/8 [0/0]

via Null0, receive

REast1#sh ip rou

O*E2  0.0.0.0/0 [110/1] via 172.31.255.253, 00:00:11, Ethernet0/0

R     172.16.0.0/16 [120/1] via 172.31.255.246, 00:00:20, Ethernet0/1

R     172.17.0.0/16 [120/1] via 172.31.255.242, 00:00:15, Ethernet0/2

R     172.18.0.0/16 [120/3] via 172.31.255.253, 00:00:17, Ethernet0/0

R     172.19.0.0/16 [120/3] via 172.31.255.253, 00:00:17, Ethernet0/0

R     172.30.0.0/16 [120/1] via 172.31.255.253, 00:00:17, Ethernet0/0

172.31.0.0/16 is variably subnetted, 10 subnets, 2 masks

R        172.31.255.228/30 [120/1] via 172.31.255.246, 00:00:20, Ethernet0/1

[120/1] via 172.31.255.242, 00:00:15, Ethernet0/2

R        172.31.255.232/30 [120/2] via 172.31.255.253, 00:00:17, Ethernet0/0

R        172.31.255.236/30 [120/2] via 172.31.255.253, 00:00:17, Ethernet0/0

C        172.31.255.240/30 is directly connected, Ethernet0/2

L        172.31.255.241/32 is directly connected, Ethernet0/2

C        172.31.255.244/30 is directly connected, Ethernet0/1

L        172.31.255.245/32 is directly connected, Ethernet0/1

O        172.31.255.248/30 [110/20] via 172.31.255.253, 00:00:11, Ethernet0/0

C        172.31.255.252/30 is directly connected, Ethernet0/0

L        172.31.255.254/32 is directly connected, Ethernet0/0

REast1# sho ipv rou

OE2 ::/0 [110/1], tag 10

via FE80::, Ethernet0/0

R   2001:DB8:ACAD:AC16::/64 [120/2]

via FE80::1:2, Ethernet0/1

R   2001:DB8:ACAD:AC17::/64 [120/2]

via FE80::1:3, Ethernet0/2

R   2001:DB8:ACAD:AC18::/64 [120/4]

via FE80::, Ethernet0/0

R   2001:DB8:ACAD:AC19::/64 [120/4]

via FE80::, Ethernet0/0

R   2001:DB8:ACAD:AC1E::/64 [120/2]

via FE80::, Ethernet0/0

L   FF00::/8 [0/0]

via Null0, receive

On RHQ0, there is no difference (see above: 2 - UE1.6) Analysis of RIP – RHQ0 IPv6 routing table), because both branch routers do not redistribute routes yet. On REast1, the default route is now learned via OSPF, for additional routing table codes see [11].

# UE1.12) Analysis Multiarea OSPF Dual Stack and Route Redistribution

a) Analyze the changes in the routing tables of RHQ0, RWest2 and RWest3.

RHQ0#sho ip rou

S*    0.0.0.0/0 [1/0] via 199.1.1.65

O E1  172.16.0.0/16 [110/110] via 172.31.255.254, 00:05:08, Ethernet0/1

O E1  172.17.0.0/16 [110/110] via 172.31.255.254, 00:05:08, Ethernet0/1

O E2  172.18.0.0/16 [110/20] via 172.31.255.250, 00:03:38, Ethernet0/2

O E2  172.19.0.0/16 [110/20] via 172.31.255.250, 00:03:28, Ethernet0/2

172.30.0.0/16 is variably subnetted, 2 subnets, 2 masks

C        172.30.0.0/16 is directly connected, Ethernet0/0

L        172.30.255.254/32 is directly connected, Ethernet0/0

172.31.0.0/16 is variably subnetted, 9 subnets, 2 masks

O E1     172.31.255.228/30 [110/110] via 172.31.255.254, 00:05:08, Ethernet0/1

O IA     172.31.255.232/30 [110/20] via 172.31.255.250, 00:04:30, Ethernet0/2

O IA     172.31.255.236/30 [110/20] via 172.31.255.250, 00:04:30, Ethernet0/2

O E2     172.31.255.240/30 [110/20] via 172.31.255.254, 00:05:22, Ethernet0/1

O E2     172.31.255.244/30 [110/20] via 172.31.255.254, 00:05:22, Ethernet0/1

C        172.31.255.248/30 is directly connected, Ethernet0/2

L        172.31.255.249/32 is directly connected, Ethernet0/2

C        172.31.255.252/30 is directly connected, Ethernet0/1

L        172.31.255.253/32 is directly connected, Ethernet0/1

199.1.1.0/24 is variably subnetted, 2 subnets, 2 masks

C        199.1.1.64/30 is directly connected, Ethernet1/0

L        199.1.1.66/32 is directly connected, Ethernet1/0

RHQ0#sho ipv rou

S   ::/0 [1/0]

via 2001:DB8:ACAD:1::1

C   2001:DB8:ACAD:1::/127 [0/0]

via Ethernet1/0, directly connected

L   2001:DB8:ACAD:1::/128 [0/0]

via Ethernet1/0, receive

OE1 2001:DB8:ACAD:AC16::/64 [110/110]

via FE80::1, Ethernet0/1

OE1 2001:DB8:ACAD:AC17::/64 [110/110]

via FE80::1, Ethernet0/1

OE2 2001:DB8:ACAD:AC18::/64 [110/20]

via FE80::1, Ethernet0/2

OE2 2001:DB8:ACAD:AC19::/64 [110/20]

via FE80::1, Ethernet0/2

C   2001:DB8:ACAD:AC1E::/64 [0/0]

via Ethernet0/0, directly connected

L   2001:DB8:ACAD:AC1E::254/128 [0/0]

via Ethernet0/0, receive

L   FF00::/8 [0/0]

via Null0, receive

RWest2#sho ip rou

O*E2  0.0.0.0/0 [110/1] via 172.31.255.237, 00:05:17, Ethernet0/1

O E1  172.16.0.0/16 [110/130] via 172.31.255.237, 00:05:17, Ethernet0/1

O E1  172.17.0.0/16 [110/130] via 172.31.255.237, 00:05:17, Ethernet0/1

172.18.0.0/16 is variably subnetted, 2 subnets, 2 masks

C        172.18.0.0/16 is directly connected, Ethernet0/0

L        172.18.255.254/32 is directly connected, Ethernet0/0

O E2  172.19.0.0/16 [110/20] via 172.31.255.237, 00:05:07, Ethernet0/1

172.31.0.0/16 is variably subnetted, 8 subnets, 2 masks

O E1     172.31.255.228/30 [110/130] via 172.31.255.237, 00:05:17, Ethernet0/1

O IA     172.31.255.232/30 [110/20] via 172.31.255.237, 00:05:17, Ethernet0/1

C        172.31.255.236/30 is directly connected, Ethernet0/1

L        172.31.255.238/32 is directly connected, Ethernet0/1

O E2     172.31.255.240/30 [110/20] via 172.31.255.237, 00:05:17, Ethernet0/1

O E2     172.31.255.244/30 [110/20] via 172.31.255.237, 00:05:17, Ethernet0/1

O IA     172.31.255.248/30 [110/20] via 172.31.255.237, 00:05:17, Ethernet0/1

O IA     172.31.255.252/30 [110/30] via 172.31.255.237, 00:05:17, Ethernet0/1

RWest2#sho ipv rou

OE2 ::/0 [110/1], tag 10

via FE80::2:1, Ethernet0/1

OE1 2001:DB8:ACAD:AC16::/64 [110/130]

via FE80::2:1, Ethernet0/1

OE1 2001:DB8:ACAD:AC17::/64 [110/130]

via FE80::2:1, Ethernet0/1

C   2001:DB8:ACAD:AC18::/64 [0/0]

via Ethernet0/0, directly connected

OE2 2001:DB8:ACAD:AC19::/64 [110/20]

via FE80::2:1, Ethernet0/1

L   2001:DB8:ACAD:AC18::254/128 [0/0]

via Ethernet0/0, receive

L   FF00::/8 [0/0]

via Null0, receive

RWest3#sho ip rou

O*E2  0.0.0.0/0 [110/1] via 172.31.255.233, 00:06:32, Ethernet0/1

O E1  172.16.0.0/16 [110/130] via 172.31.255.233, 00:06:32, Ethernet0/1

O E1  172.17.0.0/16 [110/130] via 172.31.255.233, 00:06:32, Ethernet0/1

O E2  172.18.0.0/16 [110/20] via 172.31.255.233, 00:06:32, Ethernet0/1

172.19.0.0/16 is variably subnetted, 2 subnets, 2 masks

C        172.19.0.0/16 is directly connected, Ethernet0/0

L        172.19.255.254/32 is directly connected, Ethernet0/0

172.31.0.0/16 is variably subnetted, 8 subnets, 2 masks

O E1     172.31.255.228/30 [110/130] via 172.31.255.233, 00:06:32, Ethernet0/1

C        172.31.255.232/30 is directly connected, Ethernet0/1

L        172.31.255.234/32 is directly connected, Ethernet0/1

O IA     172.31.255.236/30 [110/20] via 172.31.255.233, 00:06:32, Ethernet0/1

O E2     172.31.255.240/30 [110/20] via 172.31.255.233, 00:06:32, Ethernet0/1

O E2     172.31.255.244/30 [110/20] via 172.31.255.233, 00:06:32, Ethernet0/1

O IA     172.31.255.248/30 [110/20] via 172.31.255.233, 00:06:32, Ethernet0/1

O IA     172.31.255.252/30 [110/30] via 172.31.255.233, 00:06:32, Ethernet0/1

RWest3#sho ipv rou

OE2 ::/0 [110/1], tag 10

via FE80::2:1, Ethernet0/1

OE1 2001:DB8:ACAD:AC16::/64 [110/130]

via FE80::2:1, Ethernet0/1

OE1 2001:DB8:ACAD:AC17::/64 [110/130]

via FE80::2:1, Ethernet0/1

OE2 2001:DB8:ACAD:AC18::/64 [110/20]

via FE80::2:1, Ethernet0/1

C   2001:DB8:ACAD:AC19::/64 [0/0]

via Ethernet0/0, directly connected

L   2001:DB8:ACAD:AC19::254/128 [0/0]

via Ethernet0/0, receive

L   FF00::/8 [0/0]

via Null0, receive

| Router | Default Route code | S IPv4/6 | L IPv4/6 | C IPv4/6 | OE1 IPv4/6 | OE2 IPv4/6 | OI(A) IPv4/6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RHQ0 | S | 1/1 | 4/3 | 4/2 | 3/2 | 4/2 | 2/0 |
| RWest2 | O*E2 | 0/0 | 2/2 | 2/1 | 3/2 | 4/2 | 3/0 |
| RWest3 | O*E2 | 0/0 | 2/2 | 2/1 | 3/2 | 4/2 | 3/0 |

The OE1 routes on all three routers are coming from the redistribution from East side with metric 100 and metric type 1, metric adds up (also, the internal OSPF cost to ASBR is added with type 1 – but not with type 2) and therefor West side routers have a metric of 130 to East side networks.

The interarea routes on RHQ0 point in the direction of RWest1, where the OSPF areas 1+2 are configured. For the branch networks behind RWest1+2, there are OE2 routes.

All routes should be able to reach the four client networks

172.16/17/18/19.x.x and 2001:DB8:ACAD:AC16/17/18/19.

# UE1.14) Analysis Multiarea OSPF Dual Stack – Stub & Totally Stub Area

a) Analyze the routing tables of RWest2 and RWest3. What’s the difference? Which entries have changed and why?

RWest2#sho ip rou

O*IA  0.0.0.0/0 [110/11] via 172.31.255.237, 00:00:27, Ethernet0/1

172.18.0.0/16 is variably subnetted, 2 subnets, 2 masks

C        172.18.0.0/16 is directly connected, Ethernet0/0

L        172.18.255.254/32 is directly connected, Ethernet0/0

172.31.0.0/16 is variably subnetted, 5 subnets, 2 masks

O IA     172.31.255.232/30 [110/20] via 172.31.255.237, 00:00:27, Ethernet0/1

C        172.31.255.236/30 is directly connected, Ethernet0/1

L        172.31.255.238/32 is directly connected, Ethernet0/1

O IA     172.31.255.248/30 [110/20] via 172.31.255.237, 00:00:27, Ethernet0/1

O IA     172.31.255.252/30 [110/30] via 172.31.255.237, 00:00:27, Ethernet0/1

RWest2#sho ipv rou

OI  ::/0 [110/11]

via FE80::2:1, Ethernet0/1

C   2001:DB8:ACAD:AC18::/64 [0/0]

via Ethernet0/0, directly connected

L   2001:DB8:ACAD:AC18::254/128 [0/0]

via Ethernet0/0, receive

L   FF00::/8 [0/0]

via Null0, receive

RWest3#sho ip rou

O*IA  0.0.0.0/0 [110/11] via 172.31.255.233, 00:01:31, Ethernet0/1

172.19.0.0/16 is variably subnetted, 2 subnets, 2 masks

C        172.19.0.0/16 is directly connected, Ethernet0/0

L        172.19.255.254/32 is directly connected, Ethernet0/0

172.31.0.0/16 is variably subnetted, 2 subnets, 2 masks

C        172.31.255.232/30 is directly connected, Ethernet0/1

L        172.31.255.234/32 is directly connected, Ethernet0/1

RWest3#sho ipv rou

OI  ::/0 [110/11]

via FE80::2:1, Ethernet0/1

C   2001:DB8:ACAD:AC19::/64 [0/0]

via Ethernet0/0, directly connected

L   2001:DB8:ACAD:AC19::254/128 [0/0]

via Ethernet0/0, receive

L   FF00::/8 [0/0]

via Null0, receive

Previously (from UE1.12 analysis):

| Router | Default Route code | S IPv4/6 | L IPv4/6 | C IPv4/6 | OE1 IPv4/6 | OE2 IPv4/6 | OI(A) IPv4/6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RWest2 | O*E2 | 0/0 | 2/2 | 2/1 | 3/2 | 4/2 | 3/0 |
| RWest3 | O*E2 | 0/0 | 2/2 | 2/1 | 3/2 | 4/2 | 3/0 |

Now (after area 1 = stub, area 2 = stub no-summary):

| Router | Default Route code | S IPv4/6 | L IPv4/6 | C IPv4/6 | OE1 IPv4/6 | OE2 IPv4/6 | OI(A) IPv4/6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RWest2 | O*IA | 0/0 | 2/2 | 2/1 | 0/0 | 0/0 | 4/1 |
| RWest3 | O*IA | 0/0 | 2/2 | 2/1 | 0/0 | 0/0 | 1/1 |

RWest2 learns only LSA1-3 routes, hence, the three routes with metric 20 are still in the table.

In contrast, RWest3 does not learn routes from other areas, but only from its own area.

The ABR injects a default route in both cases (previously, this was an external route O*E2).

(pages 7-11 [13]; page 36 [14])

# References

| [1] | d. "Staff", "DN.org - Route Aggregation and CIDR," [Online]. Available: https://dn.org/route-aggregation-and-cidr/. [Accessed 31 5 2025]. |
| --- | --- |
| [2] | "skminhaj", "IT Tips for Systems and Network Administrators - Route Redistribution & Filtering Redistribution," [Online]. Available: https://skminhaj.wordpress.com/2015/03/04/route-redistribution-filtering-redistribution-distribute-lists-maps/. [Accessed 31 5 2025]. |
| [3] | P. Browning, "howtonetwork.com - Advanced Routing Protocol Design – OSPF, EIGRP and BGP," [Online]. Available: https://www.howtonetwork.com/technical/protocols/advanced-ip-routing/. [Accessed 31 5 2025]. |
| [4] | N. S. e.V., "networx security - Routing Information Protocol," [Online]. Available: https://www.networxsecurity.org/da/mitgliederbereich/glossary/r/ripng.html. [Accessed 31 5 2025]. |
| [5] | G. Malkin, "RFC 2081 - RIPng Protocol Applicability Statement," [Online]. Available: https://www.rfc-editor.org/rfc/rfc2081.html. [Accessed 31 5 2025]. |
| [6] | G. Malkin, "RFC 2080 - RIPng for IPv6," [Online]. Available: https://www.rfc-editor.org/rfc/rfc2080.html. [Accessed 31 5 2025]. |
| [7] | Huawei Technologies Co., Ltd., "NE5000E V800R024C00SPC500 Configuration Guide - Configuring IPsec Authentication for RIPng," [Online]. Available: https://support.huawei.com/enterprise/en/doc/EDOC1100419263/ad7a4c33/configuring-ipsec-authentication-for-ripng. [Accessed 31 5 2025]. |
| [8] | Huawei Technologies Co., Ltd., "NE40E-F V800R024C00SPC500 Feature Description - Understanding RIPng," [Online]. Available: https://support.huawei.cn/enterprise/en/doc/EDOC1100412634/e4d8d391/understanding-ripng. [Accessed 31 5 2025]. |
| [9] | H3C Technologies Co., Ltd., "04-Layer 3 Configuration Guide - 17-RIPng Configuration," [Online]. Available: https://www.h3c.com/en/Support/Resource_Center/HK/Home/WLAN/00-Public/Configure___Deploy/Configuration_Guides/H3C_AC_CG(E3703P61_R3509P61)-6W102/04/201905/1178348_294551_0.htm. [Accessed 31 5 2025]. |
| [10] | NetworkLessons.com, "Prefix Lists," [Online]. Available: https://notes.networklessons.com/prefix-lists. [Accessed 31 5 2025]. |
| [11] | I. Cisco Systems, "cisco E-Learning - show ip route," [Online]. Available: https://www.cisco.com/E-Learning/bulk/public/tac/cim/cib/using_cisco_ios_software/cmdrefs/show_ip_route.htm. [Accessed 31 5 2025]. |
| [12] | Cisco Systems, Inc., "FHSTP ecampus / BCN: Single-Area OSPF," [Online]. Available: https://ecampus.fhstp.ac.at/pluginfile.php/1200607/mod_resource/content/1/03_OSPF_SingleArea_2024.pdf. [Accessed 31 5 2025]. |
| [13] | Cisco Systems, Inc., "FHSTP ecampus / BCN: Multiarea OSPF," [Online]. Available: https://ecampus.fhstp.ac.at/pluginfile.php/1200608/mod_resource/content/3/04_OSPF_MultiArea_2024_v2.pdf. [Accessed 1 6 2025]. |
| [14] | Cisco Systems, Inc., "FHSTP ecampus / BCN: Advanced Single-Area OSPF Configurations," [Online]. Available: https://ecampus.fhstp.ac.at/pluginfile.php/1200609/mod_resource/content/3/05_OSPF_Advanced_2024_v2.pdf. [Accessed 1 6 2025]. |